;*==================================================

;*==================================================

;*

;* This is the Start of the EPROM

;*

;*==================================================

;*==================================================

ORG	$D000

; ----------------------

LD000:	FDB	$4A3F	; Check Sum

LD002:	FDB	$16D1	; prom id

; ----------------------

LD004:	FCB	$61	; mask id

; ----------------------

LD005:	FCB	$23	; option byte: 0010 0011

; b7: 1 = egr type

; b6: 1 =

; b5: 1 = use IAT for hot restart threshold

; b4: 1 =

; b3: 1 =

; b2: 1 =

; b1: 1 == dfco tcc unlock

; b0: 1 ==

LD006:	FCB	$11	; option byte: 0001 0001

; b7: 1 =

; b6: 1 =

; b5: 1 = reset INT on AE

; b4: 1 = do int reset on blm cell change

; b3: 0 = do inj cnst baro adj

; b2: 1 =

; b1: 1 = use rpm for tcc unlock thres (else mph)

; b0: 1 =

LD007:	FCB	$24	; option byte: 0010 0100

; b7: 1 = Shift Light (0 = tcc)

; b6: 1 = VATS enable

; b5: 1 = high gear sw polarity

; b4: 1 =

; b3: 1 =

; b2: 1 =

; b1: 1 =

; b0: 1 =

; ----------------------

LD008:	FCB	  0	; throttle kicker flag

; ----------------------

LD009:	FCB	  0	; initial SA (base timing)

; ----------------------

LD00A:	FCB	192	; max rpm for slope SA rpm / 25, 4800 rpm

LD00B:	FCB	 32	; deg / 1k rpm, VAL = N * 65536/7200, 3.5 deg/1K rpm

; ----------------------

LD00C:	FCB	240	; map filter coef, for L0028

; ----------------------

LD00D:	FCB	 57	; Main SA bias, 20 deg

LD00E:	FCB	 57	; Coolant SA bias, 20 deg

; ----------------------

LD00F:	FCB	  8	; cyl count

; ----------------------

LD010:	FDB	2458	; startup drp, 400 rpm

; ----------------------

LD012:	FCB	  6	; eng run time out param, drp's

LD013:	FCB	 40	; eng off time, (4 sec's)

; ----------------------

LD014:	FDB	  114	; max SA, 40.1deg

LD016:	FDB	65525	; min SA, -3.5deg ($FFF5)

; ----------------------

LD018:	FCB	  6	; filt coef, filter SA from startup?

; ----------------------

LD019:	FCB	 11	; 11, 3.8 deg SA max for egr

LD01A:	FCB	 12	; filter coef for egr SA off

LD01B:	FCB	  8	; filter coef for egr SA on

; ----------------------

LD01C:	FCB	240	; filter coef for rpm / 12.5, L001D/L001E

LD01D:	FCB	200	; max rpm for pseudo baro read

LD01E:	FCB	128	; 50%, min tps% for pseudo baro read

LD01F:	FCB	 10	; 3.9% max delta tps for pseudo baro read

; ----------------------

LD020:	FCB	192	; Baro filter coef

; ----------------------

LD021:	FCB	  6	; aldl sa val, 2.1 deg

; ----------------------

; Hiway quals

; ----------------------

LD022:	FCB	240	; vac, hiway SA & Fuel threshold

LD023:	FCB	 52	; mph, hiway SA threshold

LD024:	FDB	4800	; timer hiway SA, sec * 80 = 60 sec

; ----------------------

LD026:	FCB	  2	; choke SA decay delay mult

LD027:	FCB	241	; choke SA decay mult

; ----------------------

LD028:	FCB	 57	; TCC locked SA bias, 20 deg

LD029:	FCB	  6	; mph threshold for transient fuel calc interval timer (for tps/map filters)

LD02A:	FCB	 32	; 10Kpa, delta map for transient fuel calc interval timer (for tps/map filters)

LD02B:	FCB	 10	; 3Kpa, delta map thres for transient fuel calc interval timer (for tps/map filters)

; ----------------------

; hyst pairs, used in

; transient tps/rpm SA

; indexed tbl

; ----------------------

LD02C:	FCB	  3	; delta tps%, 1.2%, opening tps

LD02D:	FCB	  3	; delta tps%, 1.2%, closing tps

LD02E:	FCB	128	; tps%, 50%, opening tps

LD02F:	FCB	 26	; tps%, 10%, closing tps

LD030:	FCB	  9	; delta rpm/25 val, opening tps

LD031:	FCB	  7	; delta rpm/25 val, closing tps

; ----------------------

LD032:	FCB	 13	; delta rpm val, 325 rpm

LD033:	FCB	  3	; delta rpm val, 75 rpm

; ----------------------

; knock quals

; ----------------------

LD034:	FCB	 10	; 10 mph, knock qual thres

LD035:	FCB	 80	; 1000 rpm, knock qual thres

LD036:	FCB	126	; 50c, knock coolant qual thres

LD037:	FCB	  2	; min cnts to do retard

;*==================================================

;* Main SA Table

;*

;* Val = Spk Adv + Bias * (256/90)

;*==================================================

LD038:	FCB	  1	; flag, use map

FCB	  0	; row offset

FCB	 32	; col offset

FCB	 15	; cols per row

;    30  35  40  45  50  55  60  65  70  75  80  85  90  95 100 Kpa

FCB 105 105 105 105 105 105 105 102  94  77  64  60  48  45  42 ;  400

FCB 102 102 102 102 102 102 108 106 102  94  88  74  57  45  42 ;  600

FCB 102 105 105 105 105 105 102 102 100  94  88  71  57  48  42 ;  800

FCB 108 114 114 110 108 106 105 102  99  90  84  70  64  57  48 ; 1000

FCB 111 112 112 112 112 112 112 104 100  95  88  70  65  65  63 ; 1200

FCB 108 122 122 120 118 118 118 110 103  97  91  76  71  64  62 ; 1400

FCB 111 114 122 125 126 124 116 110 102  97  91  77  71  70  68 ; 1600

FCB 114 119 125 125 125 121 112 106 105 103  99  86  74  73  72 ; 1800

FCB 114 120 125 125 125 125 122 114 108 104  99  92  82  74  74 ; 2000

FCB 125 128 130 131 131 128 125 115 107 104 103  96  88  78  76 ; 2200

FCB 125 128 131 131 131 131 128 116 106 104 104 101  96  82  77 ; 2400

FCB 128 134 134 134 134 131 128 120 114 108 105 101  96  80  78 ; 2800

FCB 128 137 139 139 139 135 131 122 114 109 105 101  98  84  81 ; 3200

FCB 133 139 142 139 137 134 131 128 119 111 105  97  91  86  83 ; 3600

;*==================================================

;* Coolant Comp SA: VAC Vs. Deg c Cool

;*

;* Values shown include 20 deg bias

;* Added SA = Tbl - 57 * (90 / 256)

;*

;*

;* Tbl = (256/90) x Spk + coolant bias

;*==================================================

LD10E:	FCB	  0	; flag, use vac

FCB	  0	; row offset

FCB	 64	; col offset

FCB	  5	; cols per row

;     Vac    40  30  20  10   0	Cool:

FCB  68  80  80  80  86	; -40

FCB  68  80  80  80  86	;   5

FCB  57  68  74  74  86	;  25

FCB  51  57  71  71  86	;  38

FCB  45  57  63  63  71	;  50

FCB  45  57  57  60  63	;  64

FCB  57  65  68  63  63	;  80

FCB  53  48  41  45  45	; 110

FCB  10   9   8  16  16	; Hot

;*==================================================

;* Hiway Spk Adv vs Vac

;*

;* Added SA when in Hiway Mode

;*

;* Val = Spk Adv * (256/90)

;*==================================================

LD13F:

;---------------------------------

; 		Spk 	; Vac

;---------------------------------

FCB	  6	; 50

FCB	  0	; 40

FCB	  0	; 30

FCB	  0	; 20

FCB	  0	; 10

FCB	  0   	;  0

;*==================================================

;* TCC Locked Spark Retard: Rpm vs Vac

;*

;* Val = (Spk Adv + Bias) * (256/90)

;*==================================================

LD145:	FCB	  0	; flag, use vac

FCB	 32	; row offset, rpm

FCB	 64	; col offset, vac

FCB	  5	; cols per row

;   Vac:     40  30  20  10   0		; rpm

FCB  57  57  54  50  44		;  800

FCB  57  57  54  50  44		; 1200

FCB  57  57  54  50  44		; 1600

FCB  57  57  54  50  44		; 2000

;*==================================================

;* PE/WOT SA vs Baro

;*

;* Tbl = SA / 0.352

;*==================================================

LD15D:

;----------------------------------

; 	Mult 		; Baro Kpa

;----------------------------------

FCB  17		; 75	; 6 deg

FCB  14		; 85	; 5 deg

FCB  11		; 95	; 3.9 deg

FCB  11		; 105	; 3.9 deg

;*==================================================

;* SA corr idle high vs rpm / 3.125

;*

;* Val = Spk Adv * (256/90)

;*==================================================

LD161:	FCB	 96	; uppr limit, 300 rpm

;---------------------------------

; 		Spk 	; rpm

;---------------------------------

FCB	  3	; 0

FCB	 21	; 50

FCB	 21	; 100

FCB	 18	; 150

FCB	 15	; 200

FCB	 15	; 250

FCB	 15	; 300

;*==================================================

;* SA corr idle high multplier vs s/d map

;*

;* Val = Mult / 256

;*==================================================

LD169:	FCB	 96	; uppr limit, 50 map

;---------------------------------

; 		Mult	; Map

;---------------------------------

FCB	255	; 20

FCB	255	; 25

FCB	200	; 30

FCB	175	; 35

FCB	160	; 40

FCB	 64	; 45

FCB	 64	; 50

;-----------------------------------------------

;-----------------------------------------------

LD171:	FCB	  0	;

;*==================================================

;* SA corr idle low vs rpm / 3.125

;*

;* Val = Spk Adv * (256/90)

;*==================================================

LD172:	FCB	 96	; uppr limit, 300 rpm

;---------------------------------

; 		Spk 	; rpm

;---------------------------------

FCB	  3	; 0

FCB	 15	; 50

FCB	 18	; 100

FCB	 15	; 150

FCB	 15	; 200

FCB	  9	; 250

FCB	  6	; 300

;*==================================================

;* SA corr idle low multplier vs s/d map

;*

;* Val = Mult / 256

;*==================================================

LD17A:	FCB	 96	; uppr limit, 50 map

;---------------------------------

; 		Mult	; Map

;---------------------------------

FCB	  0	; 20

FCB	 80	; 25

FCB	120	; 30

FCB	160	; 35

FCB	200	; 40

FCB	255	; 45

FCB	255	; 50

;-----------------------------------------------

;-----------------------------------------------

LD182:	FCB	  0	;

;*==================================================

;* Choke SA vs Cool

;*

;* Initial Sa Which Is Decayed In Some Time Period

;*

;* Val = Spk Adv * (256/90)

;*==================================================

LD183:

FCB 4		; 5 Line Table

;---------------------------------

; 	Deg's 		; Deg c Cool

;---------------------------------

FCB 14		;  COLD

FCB 10		;  24.0

FCB 14		;  49.7

FCB 14		;  80.5

FCB 14		;  HOT

;*==================================================

;* Choke Spk Time Out Decay Delay vs Cool

;*

;* Tbl = Seconds

;*==================================================

LD189:

FCB 8		; 9 line table

;---------------------------------

; 	Sec's 		; Deg C

;---------------------------------

FCB 10		; COLD

FCB  5		;

FCB  0		; 24.0

FCB  5		;

FCB 10		; 49.7

FCB 20		;

FCB 20		; 80.5

FCB 20		;

FCB 20		; HOT

;*==================================================

;* Max Knock Retard SA

;*

;* PE mode vs Rpm

;*

;*==================================================

LD193:

FCB	80	; upr rpm limit

;---------------------------------

;	pct/msec 	; RPM

;---------------------------------

FCB  52		;  400

FCB  74		;  800

FCB  74		; 1200

FCB  74		; 1600

FCB  74		; 2000

;*==================================================

;* Max Knock Retard SA

;*

;* non-PE mode vs Vac

;*

;*==================================================

LD199:

FCB	128	; upr vac limit

;---------------------------------

;	pct/msec 	; Vac

;---------------------------------

FCB  85		; 40

FCB  68		; 30

FCB  68		; 20

FCB  68		; 10

FCB  80		;  0

;*==================================================

;* IAT max knock retard SA multiplier

;*

;* tbl = Mult * 128

;*==================================================

LD19F:

;---------------------------------

;	Mult		; IAT c

;---------------------------------

FCB	128	; -40

FCB	128	;   5

FCB	128	;  23

FCB	128	;  38

FCB	128	;  50

FCB	141	;  62

FCB	154	;  80

FCB	154	; 107

FCB	154	; 200

;*==================================================

;* Spk Latencies Correction vs Rpm

;*

;* 0 - 4800 RPM

;*

;* tbl = usec/15.26

;*==================================================

LD1A8:

FCB 192		; upper rpm tbl limit

;---------------------------------

; 	usec 		; rpm

;---------------------------------

FCB 0		; 0

FCB 0		; 400

FCB 14		; 800

FCB 18		; 1200

FCB 20		; 1600

FCB 16		; 2000

FCB 18		; 2400

FCB 16		; 2800

FCB 20		; 3200

FCB 21		; 3600

FCB 19		; 4000

FCB 20		; 4400

FCB 20		; 4800

;*==================================================

;* Retard Attack Rate vs Rpm: PE mode

;*

;* Deg/msec

;* Tbl = (deg/ms)/.0255

;*==================================================

LD1B6:

FCB	80	; upr rpm limit

;---------------------------------

;	pct/msec 	; RPM

;---------------------------------

FCB   8		;  400

FCB  12		;  800

FCB  24		; 1200

FCB  36		; 1600

FCB  40		; 2000

;*==================================================

;* Retard Attack Rate vs Rpm: non-PE mode

;*

;* Deg/msec

;* Tbl = (deg/ms)/.0255

;*==================================================

LD1BC:

FCB	128	; upr vac limit

;---------------------------------

;	pct/msec 	; Vac

;---------------------------------

FCB  10		; 40

FCB   8		; 30

FCB   9		; 20

FCB  10		; 10

FCB  11		;  0

;*==================================================

;* Retard Recovery Rate vs Rpm: PE mode

;*

;* Tbl = (%/ms) * (256/500)

;* Tbl = 0.512 * %/ms

;*==================================================

LD1C2:

FCB	64	; upr rpm limit

;---------------------------------

;	pct/msec 	; RPM

;---------------------------------

FCB  42		;  400

FCB  32		;  800

FCB  19		; 1200

FCB  10		; 1600

;*==================================================

;* Retard Recovery Rate vs Vac: non-PE mode

;*

;* Tbl = (%/ms) * (256/500)

;* Tbl = 0.512 * %/ms

;*==================================================

LD1C7:

FCB	120	; upr vac limit

;---------------------------------

;	pct/msec 	; Vac

;---------------------------------

FCB  30		;

FCB  24		;

FCB  18		;

FCB  13		;

;-----------------------------------------------

;-----------------------------------------------

LD1CB:	FCB	  0	;

;*==================================================

;* 2d lkup

;*

;* Base SA for Transient SA vs delta map

;*==================================================

LD1CC:

;---------------------------------

;		SA	; delta map

;---------------------------------

FCB	 64	;   0

FCB	160	;  10

FCB	224	;  20

FCB	255	;  30

FCB	255	;  40

FCB	255	;  50

FCB	255	;  60

FCB	255	;  70

;*==================================================

;*

;* MAP mult for trans SA vs s/d map cnts

;*

;* tbl = mult * 256

;*==================================================

LD1D5:

;---------------------------------

;		mult	; s/d map

;---------------------------------

FCB	  0	;  20

FCB	  0	;  25

FCB	  0	;  30

FCB	  0	;  35

FCB	  0	;  40

FCB	  0	;  45

FCB	  8	;  50

FCB	 14	;  55

FCB	 14	;  60

FCB	 14	;  65

FCB	 14	;  70

FCB	 14	;  75

FCB	 14	;  80

FCB	 11	;  85

FCB	 11	;  90

FCB	  9	;  95

FCB	  9	; 100

;*==================================================

;* delta tps & rpm SA term mult vs delta rpm / 25

;*

;* Indexed Lookup vs delta rpm/25 value

;*

;* SA mult of next table

;*

;* tbl = mult * 64

;*==================================================

LD1E6:

;----------------------------------

; 		SA	; delta rpm

;----------------------------------

FCB	240	;   0

FCB	243	;  25

FCB	246	;  50

FCB	248	;  75

FCB	250	; 100

FCB	250	; 125

FCB	250	; 150

FCB	250	; 175

FCB	  0	; 200

FCB	247	; 225

FCB	  6	; 250

FCB	  8	; 275

FCB	 10	; 300

FCB	 13	; 325

FCB	 16	; 350

FCB	 18	; 375

FCB	 20 	; 400

;*==================================================

;* delta tps & rpm SA term vs rpm / 25

;*

;* 2d lkup w/upr limt in 1st tbl entry, lwr limt in B

;* by rpm / 25

;*==================================================

LD1F7:	FCB	128	; uppr rpm/25 limit

;----------------------------------

; 	Mult 		; rpm

;----------------------------------

FCB	 96	;  400

FCB	 96	;  800

FCB	 96	; 1200

FCB	 64	; 1600

FCB	 48	; 2000

FCB	 32	; 2400

FCB	 24	; 2800

FCB	 18	; 3200

;*==================================================

;* IAT mult for trans SA vs iat adc cnts

;*

;* tbl = mult * 256

;*==================================================

LD200:	FCB	  4	; line cnt

;----------------------------------

; 	Mult 		; iat

;----------------------------------

FCB	  0	; 200c

FCB	 64	;  80c

FCB	176	;  48c

FCB	255	;  23c

FCB	255	; -40c

;*==================================================

;* Baro Correction vs Tps

;*

;* Used to correct Pseudo baro read

;* TBL = N * 2.7 * (256/40)

;* TBL = N * 17.28

;*

;* Kpa/1K RPM

;*==================================================

LD206:

FCB 4		; 5 Line Table

;---------------------------------

; 	Kpa/1K RPM 	; %TPS

;---------------------------------

FCB 100		;   0

FCB 100		;  25

FCB  95		;  50

FCB  30		;  75

FCB   5		; 100

;*==================================================

;* EGR Quals

;*

;* Hyst pairs

;* 11 byte indexed table

;*==================================================

LD20C:	FCB	177	; 30c, coolant thres

LD20D:	FCB	165	; 35c, coolant thres

LD20E:	FCB	  0	; 0, mph thres

LD20F:	FCB	  0	; 0, mph thres

LD210:	FCB	  5	; 2%, tps thres

LD211:	FCB	  9	; 3.5%, tps thres

LD212:	FCB	 16	; 25 Kpa, map thres

LD213:	FCB	 26	; 28 Kpa, map thres

;

LD214:	FCB	128	; 50%, tps thres

LD215:	FCB	196	; 76%, tps thres

LD216:	FCB	207	; 80% tps thres

; ----------------------

LD217:	FCB	1279	; $4FF, egr something or other

;*==================================================

;*

;*==================================================

LD219:	FCB	117	; scalar to convert from tps adc to tps%

LD21A:	FCB	 51	; tps idle setting init value

LD21B:	FCB	  1	; tps idle setting filter coeficient

;*==================================================

;*

;* Closed loop stuff

;*

;*==================================================

LD21C:	FCB	240	; O2 filter coef, minor loop

LD21D:	FCB	138	; O2 init value, 599mV

LD21E:	FCB	 32	;

LD21F:	FCB	 48	; tps% thres

LD220:	FCB	128	; tps% thres

LD221:	FCB	  4	; tps% delta thres

LD222:	FCB	  8	; IAC opening additional AE bpw, 120usc

LD223:	FCB	  0	; timer/counter for transient fuel calc interval timer (for tps/map filters)

LD224:	FCB	 16	; tps filter coef for L0045

LD225:	FCB	 64	; coolant threshold for following filt coef's

LD226:	FCB	 20	; filter coef for L002C, s/d map term

LD227:	FCB	 36	; filter coef for L002C, s/d map term

; ----------------------

LD228:	FCB	199	; coolant val, 20c, adc cnts, HiWay SA & Fuel threshold

; also for closed loop hot/cold timer threshold

; ----------------------

LD229:	FCB	 15	; closed loop eng run threshold, hot

LD22A:	FCB	200	; closed loop eng run threshold, cold

LD22B:	FCB	128	; closed loop coolant thres, 48c

; ----------------------

LD22C:	FCB	 50	; O2 sensor ready timeout, exit clsd lp

; ----------------------

; blm enable thresholds

; ----------------------

LD22D:	FCB	189	; 25c, min cool thres

LD22E:	FCB	  0	; 200c, max cool thres

LD22F:	FCB	 20	; 26 Kpa, min s/d map thres

LD230:	FCB	255	; 100 Kpa, max s/d map thres

LD231:	FCB	240 	; 6000 rpm, max rpm

; ----------------------

LD232:	FCB	147	; stoich

LD233:	FCB	 18	; idle afr vs Vac (1.8 afr), used instead of afr vs vac

; ----------------------

LD234:	FCB	 52	; hiwayfuel mph threshold, 52

LD235:	FCB	166	; cmp'd against a clsd lp term used in egr test

LD236:	FCB	100	; hiway fuel lockout timer val

LD237:	FCB	255	; hiway fuel timer val, stay in

; ----------------------

LD238:	FCB	 12	; blm update time

LD239:	FCB	 32	; blm cell boundry, rpm / 25, 800 rpm

LD23A:	FCB	 64	; blm cell boundry, rpm / 25, 1600 rpm

LD23B:	FCB	 96	; blm cell boundry, rpm / 25, 2400 rpm

LD23C:	FCB	 30	; blm cell boundry, s/d map, 29.4 Kpa

LD23D:	FCB	112	; blm cell boundry, s/d map, 55.0 Kpa

LD23E:	FCB	176	; blm cell boundry, s/d map, 75.0 Kpa

LD23F:	FCB	  2	; blm cell hyst, rpm / 25, 50 rpm

LD240:	FCB	  6	; blm cell hyst, s/d map, 1.9Kpa

; ----------------------

LD241:	FCB	135	; Injector Constant

LD242:	FCB	 32	; inj cnst filter coef for egr off

LD243:	FCB	 28	; inj cnst filter coef for egr on

; ----------------------

LD244:	FCB	  3	; INT delta from 128 to move blm cell

LD245:	FCB	  1	; amount to move blm for adjustment

; ----------------------

LD246:	FCB	150	; max blm

LD247:	FCB	108	; min blm

; ----------------------

; O2 State Limits

; Val = N * 230.4

; mV = Val * 4.3403

; ----------------------

LD248:	FCB	187	; Hi O2 to stay closed loop, 811 mV

LD249:	FCB	 84	; Lo O2 to stay closed loop, 365 mV

LD24A:	FCB	128	; Hi O2 to go closed loop, 556 mV

LD24B:	FCB	 56	; Lo O2 to go closed loop, 243 mV

; ----------------------

LD24C:	FCB	132	; use when in idle, instead of indx lkup vs airflow

LD24D:	FCB	159	; use when in idle

LD24E:	FCB	160	; use when in idle

; ----------------------

LD24F:	FCB	 24	; cls lp decel, enlean map thresh. (26 kpa)

LD250:	FCB	 40	; cls lp decel, enlean rpm thresh. (1000 rpm)

; ----------------------

LD251:	FCB	 40	; minimum INT

LD252:	FCB	158 	; maximum INT

; ----------------------

LD253:	FCB	200	; max lean afr, 20:1

; ----------------------

LD254:	FCB	205	; clear flood tps% threshold, 80%

LD255:	FCB	200	; clear flood afr, 20:1

; ----------------------

LD256:	FCB	241	; cranking coolant threshold, -10c

; ----------------------

LD257:	FCB	 23	; coolant for tps% tbl threshold, 120c

LD258:	FCB	  8	; tps% hyst to stay in PE

LD259:	FCB	112	; coolant threshold to enrich PE afr, ~55c

LD25A:	FCB	 10	; make pe richer by this afr * 10

; ----------------------

LD25B:	FCB	  9	; delay mult to decay choke afr

LD25C:	FCB	 25	; inital delay from eng strt to decay choke afr

LD25D:	FCB	152	; cool/iat threshold, 40c

; ----------------------

LD25E:	FCB	 72	; hot restart iat | coolant threshold, 75c

LD25F:	FCB	  2	; added iac steps for hot restart

LD260:	FCB	 35	; addtional choke afr for hot-restart (3.5:1)

; ----------------------

LD261:	FCB	255	; min mph threshold to restore fuel

LD262:	FCB	255	; max mph threshold to kill fuel

; ----------------------

LD263:	FDB	 65	; max sync bpw to exit async fuel, 992usec

LD265:	FDB	 45	; min sync bpw to enter async fuel, 687usec

LD267:	FDB	 26	; injector bias, added to bpw, 396usec

LD269:	FDB	786	; max async bpw, 12msec

LD26B:	FDB	 45	; min async bpw, 687usec

; ----------------------

; decel enleanment quals

; ----------------------

LD26D:	FCB	  3	; delta tps% required for decel enleanment

LD26E:	FCB	  8	; delta s/d map required for decel enleanment

LD26F:	FCB	224	; decel enleanment factor

; ----------------------

; decel fuel cut off quals

; ----------------------

LD270:	FCB	 54	; rpm limit for not in dfco, 1350 rpm

LD271:	FCB	 34	; rpm limit for in dfco, 850

LD272:	FCB	 40	; mph limit for not in dfco

LD273:	FCB	 35	; mph limit for in dfco

LD274:	FCB	 34	; map limit, lower (adc cnts) 23 KPa = (ADC * 0.369) + 10.415

LD275:	FCB	 66   	; map limit, upper (adc cnts) 35 KPa

LD276:	FCB	  4	; delta rpm thres for dfco

LD277:	FCB	 10	; delta map threshold, 23KPa

LD278:	FDB	262	; fuel pulse when exiting dfco. 15.26usec

LD27A:	FCB	 80	; dfco timer

LD27B:	FCB	160	; ??? msec, after decel fuel cut off for bpw

; val = msec x 0.80

LD27C:	FCB	  8	; tps% dfco threshold

LD27D:	FCB	  1	; 10.2% mult for bpw in decel fuel c/o

LD27E:	FCB	231	; dfco coolant threshold, 0c

; ----------------------

; ----------------------

LD27F:	FCB	 36	; 156 mvdc, O2 window for fast O2 r/l

LD280:	FCB	  0	; O2 err val threshold for int & prop timer resets

; ----------------------

; values for gms/sec calc

; ----------------------

LD281:	FCB	120	; scalar for gms/sec calc

LD282:	FCB	 18	; offset for map term

LD283:	FCB	164	; max for rpm term

LD284:	FCB	100	; max for map term

; ----------------------

LD285:	FCB	  7	; mph threshold for in idle detection

; ----------------------

; prop stuff

; ----------------------

LD286:	FCB	  9	; prop duration for in idle

LD287:	FCB	  4	; prop gain multiplier for idle

LD288:	FCB	  1	; prop gain multiplier for idle

LD289:	FCB	 10	; mean R/L O2 threshold for idle

LD28A:	FCB	 14	; mean R/L O2 threshold for idle

LD28B:	FCB	232	; O2 err term multiplier for rich O2

LD28C:	FCB	208	; O2 err term multiplier for in idle

; ----------------------

LD28D:	FCB	  0	; coolant threshold for status bits b6 & b5 of L0003

LD28E:	FCB	 25	; an afr val

LD28F:	FCB	  2	; an afr timer val

LD290:	FCB	 13	; an afr sub'd val

; ----------------------

LD291:	FDB	$FFFF	; vats freq

LD293:	FDB	$FFFF	; vats freq

;*==================================================

;* Accel Enrich vs Diff Map

;*

;* Async Only To Stretch Bpw

;* Table = msec * 16.384

;*==================================================

LD295:

FCB 4		; 5 lines in table

;---------------------------------

; 	msec 		; Kpa MAP

;---------------------------------

FCB  3		;   0

FCB  6		;  20

FCB 14		;  40

FCB 18		;  60

FCB 32		;  80

;*==================================================

;* Accel Enrich vs Diff Tps

;*

;* Async Only To Stretch Bpw

;* Tbl = 16.384 * msec

;* Tbl = 0.016384 * usec

;*

;* Added to delta map AE & IAC opening

;*==================================================

LD29B:

FCB 4		; 5 lines in table

;---------------------------------

; 	usec 		; Diff %TPS

;---------------------------------

FCB  9		;  0.0

FCB 10		;  3.1

FCB 11		;  6.2

FCB 18		;  9.4

FCB 36		;  12.5

;*==================================================

;*

;* EGR Injector Constant vs. Vac

;*

;*==================================================

LD2A1:

FCB	176	; upper table limit

;---------------------------------

; 		cnst	; vac

;---------------------------------

FCB	135	;

FCB	133	;

FCB	133	;

FCB	136	;

FCB	121	;

FCB	121	;

FCB	121	;

FCB	120	;

FCB	120	;

FCB	121	;

FCB	120	;

FCB	120	;

;*==================================================

;*

;* VE%, FL1

;*

;*==================================================

LD2AE:	FCB	  0	; row offset, map

FCB	 16	; col offset, rpm

FCB	  8	; col's per row

LD2B1:

; ------------------------------------------------------------

; Rpm:      400 800 1200 1600 2000 2400 2800 3200	; Map:

; ------------------------------------------------------------

FCB  50  62   58   59   66   80   75   78	;  20

FCB  48  75   80   80   90   89   90   99	;  30

FCB  71  88   90   95  100  101  109  112	;  40

FCB  85 103  102  101  107  108  114  122	;  50

FCB 110 115  110  107  112  118  120  126	;  60

FCB 122 121  114  110  115  116  122  130	;  70

FCB 124 126  116  112  116  119  120  128	;  80

FCB 135 128  112  108  112  116  120  128	;  90

FCB 140 135  115  100  101   99   94  100	; 100

;*==================================================

;*

;* VE% vs rpm / 25, FL2

;*

;* Stored in L0050, added to FL1

;*

;*==================================================

LD2F9:

; ----------------------------

;		VE	; Rpm

; ----------------------------

FCB	 64	;    0

FCB	 66	;  400

FCB	 74	;  800

FCB	 96	; 1200

FCB	112	; 1600

FCB	116	; 2000

FCB	118	; 2400

FCB	117	; 2800

FCB	112	; 3200

FCB	102	; 3600

FCB	100 	; 4000

FCB	 94	; 4400

FCB	 96	; 4800

FCB	 88	; 5200

FCB	 76	; 5600

FCB	 68	; 6000

FCB	 60	; 6375

;*==================================================

;* 2d lkup vs. coolant adc

;*

;* inverse coolant term for BPW calc

;*

;*==================================================

LD30A:

;----------------------------------------------

;	BIN VAL		; Deg K    Deg C  Deg F

;----------------------------------------------

FCB 107		; 423K       HOT   HOT

FCB 122		; 410      138.0c  280f

FCB 132		; 379      107.0   225

FCB 137		; 365       91.0   196

FCB 142		; 352       80.0   176*

FCB 145		; 345       71.0   160

FCB 149		; 336       63.0   145

FCB 152		; 329       56.0   133

FCB 155		; 323       49.3   121

FCB 158		; 317       43.3   110

FCB 161		; 311       37.0    99

FCB 165		; 303       30.5    87

FCB 169		; 296       23.5    74**

FCB 173		; 289       15.5    60

FCB 179		; 279        6.0    43

FCB 189		; 265       -8.5    17

FCB 211		; 237K      COLD  COLD

;*==================================================

;* Voltage Comp vs Battery (FuelPump) for BPW calc

;*

;* Tbl = Mult * 128

;*==================================================

LD31B:

;---------------------------------

;	Mult		; %     PmpV

;---------------------------------

FCB 128		; 1.000 0.0

FCB 128		; 1.000 1.6

FCB 255		; 1.250 3.2

FCB 255		; 1.250 4.8

FCB 235		; 1.148 6.4

FCB 176		; 1.148 8.0

FCB 145		; 1.100 9.6

FCB 129		; 1.100 11.2

FCB 128		; 1.047 12.8

FCB 126		; 1.000 14.4*

FCB 126		; 0.984 16.0

FCB 122		; 0.819 17.6

FCB 120		; 0.938 19.2

FCB 118		; 0.938 20.8

FCB 116		; 0.906 22.4

FCB 114		; 0.891 24.0

FCB 112		; 0.875 25.5

;*==================================================

;* Decel Coolant Factor vs Coolant Temp

;*

;* Table = factor * 32

;*==================================================

LD32C:

FCB 4		; 5 line table

;---------------------------------

; 	MULT 		; Deg c

;---------------------------------

FCB  96		;  COLD

FCB  80		;  24.0

FCB  64		;  49.0

FCB  64		;  80.5

FCB  64		;  HOT

;*==================================================

;* Accel Enrich vs Coolant

;*

;* Used as a multiplier of final TPS & MAP PW val

;*

;* Tbl = 32 * Factor

;*==================================================

LD332:

;----------------------------------

; Factor Cool 		deg c

;----------------------------------

FCB 208		;  COLD

FCB 192		;  -8.5

FCB 181		;   6.0

FCB 160		;  15.5

FCB 143		;  23.5

FCB 131		;  30.5

FCB 120		;  37.0

FCB 109		;  43.3

FCB  96		;  49.3

FCB  86		;  56.0

FCB  77		;  63.0

FCB  64		;  71.0

FCB  56		;  80.0

FCB  44		;  91.0

FCB  40		; 107.0

FCB  40		; 138.5

FCB  36		;   HOT

;*==================================================

;* Open Loop AFR vs Vacuum

;*

;* Sum Vac Afr (LAfrVac) + Cool Afr (LAfrCool)

;*

;* Tbl = AFR * 10

;*==================================================

LD343:

FCB 8		; 9 line table

;---------------------------------

; 	AFR 		; VAC Kpa

;---------------------------------

FCB 22		; 80

FCB 24		; 70

FCB 24		; 60

FCB 24		; 50

FCB 20		; 40

FCB 20		; 30

FCB 12		; 20

FCB  7		; 10

FCB  5		;  0

;*==================================================

;* Choke Afr Vs. Cool

;*

;* Sub'd from desired afr to make richer

;*

;* Table = afr * 10

;*==================================================

LD34D:

;----------------------------------

; 	MULT 		; Deg c Cool

;----------------------------------

FCB  35		;  COLD

FCB  30		;  -7.0

FCB  30		;   6.5

FCB  25		;  16.0

FCB  20		;  24.0

FCB  16		;  31.0

FCB  21		;  37.3

FCB  21		;  43.7

FCB  21		;  49.7

FCB  21		;  56.5

FCB  18		;  63.5

FCB  16		;  71.5

FCB  15		;  80.5

FCB  10		;  92.0

FCB  10		; 108.5

FCB  10		; 138.5

FCB  10		;   HOT

;*==================================================

;* Time Out Decay Mult vs Cool

;* (choke Afr Mult Vs. Cool)

;*

;* After Start Up Afr,

;* Increases With Time, Simulating A Choke.

;*

;* Table = Factor * 256

;*==================================================

LD35E:

;----------------------------------

; 	MULT 		; Deg c Cool

;----------------------------------

FCB 248		;  COLD

FCB 248		;  -7.0

FCB 248		;   6.5

FCB 248		;  16.0

FCB 240		;  24.0

FCB 230		;  31.0

FCB 248		;  37.3

FCB 248		;  43.7

FCB 248		;  49.7

FCB 248		;  56.5

FCB 248		;  63.5

FCB 230		;  71.5

FCB 224		;  80.5

FCB 192		;  92.0

FCB 192		; 108.5

FCB 192		; 138.5

FCB 192		;   HOT

;*==================================================

;* Crank Afr vs Coolant

;*

;* Afr During Crank

;*

;* Tbl = AFR * 10

;*==================================================

LD36F:

;----------------------------------

; 	AFR 		; Deg c Cool

;----------------------------------

FCB  4		;   COLD

FCB 20		;   -7.0

FCB 25		;    6.5

FCB 50		;   16.0

FCB 64		;   24.0

FCB 64		;   31.0

FCB 64		;   37.3

FCB 64		;   43.7

FCB 64		;   49.7

FCB 64		;   56.5

FCB 64		;   63.5

FCB 64		;   71.5

FCB 64		;   80.5

FCB 64		;   92.0

FCB 80		;  108.5

FCB 80		;  138.5

FCB 64		;    HOT

;*==================================================

;* Cold Engine Crank AFR multiplier vs Vac

;*

;*

;* Cold crank AFR Mult? for cts < -10c vs Vac

;*

;*==================================================

LD380:

;----------------------------------

; 	AFR mult	; Vac

;----------------------------------

FCB	13	;

FCB	15	;

FCB	16	;

FCB	20	;

FCB	30	;

FCB	28	;

FCB	26	;

FCB	24	;

FCB	22	;

FCB	21	;

FCB	15	;

;*==================================================

;* Open Loop AFR vs Coolant

;*

;* Sum Vac Afr (LAfrVac) + Cool Afr (LAfrCool)

;*

;* Tbl = AFR * 10

;*==================================================

LD38B:

;----------------------------------

; 	AFR 		; Deg c Cool

;----------------------------------

FCB  80		;   COLD

FCB 103		;   -7.0

FCB 105		;    6.5

FCB 110		;   16.0

FCB 118		;   24.0

FCB 122		;   31.0

FCB 132		;   37.3

FCB 133		;   43.7

FCB 133		;   49.7

FCB 133		;   56.5

FCB 127		;   63.5

FCB 127		;   71.5

FCB 127		;   80.5

FCB 127		;   92.0

FCB 122		;  108.5

FCB 116		;  138.5

FCB 116		;    HOT

;*==================================================

;* Async Mult vs Rpm Table

;*

;*

;* This Mult Is Used To Convert Sync PW

;* (pw/cyl) to Async (pw/12.5 Msec) Inj.

;* For Async Injections

;*

;* (note: not used for AE)

;*

;* ASync Is Both Injectors @ 80Hz,

;* Sync Is Alternating Injections

;*

;* Factor = (Cyls/2) * (rpm/60) * (0.0125/2)

;*

;* .0125 = Pulse Period, (80 Hz)

;*

;*

;* Tbl = Mult * 64

;*==================================================

LD39C:

FCB 8		; 9 Line Table

;----------------------------------

; 	Mult 		;  rpm

;----------------------------------

FCB   0		;    0

FCB  21		;  800

FCB  43		; 1600

FCB  64		; 2400

FCB  85		; 3200

FCB 107		; 4000

FCB 128		; 4800

FCB 149		; 5600

FCB 171		; 6400

;*==================================================

;* BPW Injector Constant Mult vs iat

;*

;* Tbl = Mult * 128

;*==================================================

LD3A6:

;----------------------------------

; 	Mult 		; iat

;----------------------------------

FCB 131		;  48c

FCB 136		;  63c

FCB 148		;  80c

FCB 134 	;  110c

;*==================================================

;* BPW Injector Constant Mult vs Baro

;*

;* Tbl = Mult * 128

;*==================================================

LD3AA:

;----------------------------------

; 	Mult 		; Baro Kpa

;----------------------------------

FCB 134		; 75

FCB 130		; 85

FCB 128		; 95

FCB 128		; 105

;*==================================================

;* Tps Mult vs. Baro

;*

;* tbl = factor * 255

;*==================================================

LD3AE:

;----------------------------------

; 	MULT 		; Kpa BARO

;----------------------------------

FCB 213		;  75

FCB 213		;  85

FCB 255		;  95

FCB 255		; 105

;*==================================================

;* Tps vs Rpm Thresh For Wot Enable

;*

;* Low coolant tbl

;* Table = % * 2.55

;*==================================================

LD3B2:

;----------------------------------

; 	tps 		; RPM

;----------------------------------

FCB  98		;

FCB 115		;

FCB 128		;

FCB 128		;

FCB 128		;

;*==================================================

;* Wot Fuel Afr Vs. Rpm

;*

;* Tbl = AFR * 10

;*==================================================

LD3B7:

;----------------------------------

; 	    AFR 	; RPM

;----------------------------------

FCB 124		;  800

FCB 111		; 1600

FCB 118		; 2400

FCB 106		; 3200

FCB 120		; 4000

;*==================================================

;* HiWay Mode AFR vs Vac

;*

;* tbl = afr * 10

;*==================================================

LD3BC:

;----------------------------------

;		Afr	; Vac, Kpa

;----------------------------------

FCB	150	; 80

FCB	150	; 70

FCB	166	; 60

FCB	166	; 50

FCB	160	; 40

FCB	155	; 30

FCB	150	; 20

FCB	144	; 10

FCB	144 	;  0

;*==================================================

;* Tps vs Rpm Thresh For Wot Enable

;*

;* High coolant tbl

;* Table = % * 2.55

;*==================================================

LD3C5:

;----------------------------------

; 	tps%		; RPM

;----------------------------------

FCB 128		;  400

FCB 128		; 1200

FCB  77		; 1600

FCB  77		; 3200

FCB  77		; 4800

;-----------------------------------------------

; 2d lkup's:

;

; Proportional & int stuff

; Table lookups are by gms/sec airflow

;

; L004D: mean R/L O2 threshold

; L004E: rich O2 threshold

; L004F: lean O2 threshold

; L00AF: INT delay

;-------------------------------------------------

;*==================================================

;* Integrator Delay vs Air Flow

;* 5 Line Tbl, Use Air Flow 0-64 Bin

;*

;* Mult x Int Delay Mult (vs. Slo O2) * LIntDlyMult

;*

;* tbl = Sec's * 40

;*    = msec * 0.04

;*

;* msec = val / 0.04 = val * 25

;* (stored in L00AF)

;*==================================================

LD3CA:

;----------------------------------

;	 msec's 	; air flow

;----------------------------------

FCB 22		;  0

FCB 18		; 16

FCB 12		; 32

FCB  8		; 48

FCB  6		; 64

;*==================================================

;* Mean Rich/Lean Threshold For Unfiltered O2

;* As A Function Of Flow

;*

;* mV = val * 4.34

;* Tbl = 230.4 * Vdc

;* (stored in L004D)

;*==================================================

LD3CF:

;----------------------------------

; 	mvdc 		; air flow

;----------------------------------

FCB 120		;  0

FCB 120		; 16

FCB 134		; 32

FCB 134		; 48

FCB 130		; 64

;*==================================================

;* Rich O2 Threshold as Funct Of Air Flow

;*

;* mV = val * 4.34

;* Tbl = 230.4 * Vdc

;* (stored in L004E)

;*==================================================

LD3D4:

;----------------------------------

; 	mvdc 		; air flow

;----------------------------------

FCB 135		;  0

FCB 135		; 16

FCB 138		; 32

FCB 138		; 48

FCB 132		; 64

;*==================================================

;* Lean O2 Threshold as Funct Of Air Flow

;*

;* mV = val * 4.34

;* Tbl = 230.4 * Vdc

;* (stored in L004F)

;*==================================================

LD3D9:

;----------------------------------

; 	mvdc 		; air flow

;----------------------------------

FCB 120		;  0

FCB 120		; 16

FCB 132		; 32

FCB 128		; 48

FCB 128		; 64

;*==================================================

; **** End of table indexing

;*==================================================

;*==================================================

;* 2d lkup

;*

;* O2 filter coef vs airflow (gms/sec)

;* Used for L003E

;*==================================================

LD3DE:

;----------------------------------

; 	     Coef	; air flow

;----------------------------------

FCB   4		;  0

FCB   6		; 16

FCB  11		; 32

FCB  18		; 48

FCB  22		; 64

;*==================================================

;* Proportional Value vs Slo O2 Error

;*

;* For Selecting Bin Val To Adjust Proportional Gains

;*

;*

;* Tbl = Bin * 1

;*

;* The ERR value used for the lookup is from

;* the O2 ERR VAL, this is calc'd from the SLO

;* FILT O2 & R/L O2 thresholds. It is the term

;* of how far from desired we are.

;*==================================================

LD3E3:

;----------------------------------

; 	Bin 		; O2 Err

;----------------------------------

FCB  18		;   0

FCB  18		;   8

FCB  18		;  16

FCB  12		;  24

FCB  12		;  32

FCB  20		;  40

FCB  24		;  48

FCB  36		;  56

FCB  40		;  64

FCB  48		;  72

FCB  48		;  80

FCB  48		;  88

FCB  48		;  96

;*==================================================

;* Proportional Term Duration vs. O2 Error

;*

;* For Selecting Sec's To Allow Adjust To Take Affect

;*

;* Tbl = Sec's * 40

;* msec * 0.04

;*==================================================

LD3F0:

;----------------------------------

; 	msec's 		; O2 Error

;----------------------------------

FCB   2		;  0

FCB   3		;  8

FCB   4		; 16

FCB   8		; 24

FCB  16		; 32

FCB  32		; 40

FCB  64		; 48

FCB  96		; 56

FCB 120		; 64

FCB 120		; 72

FCB 120		; 80

FCB 120		; 88

FCB 120		; 96

;*==================================================

;* Proportional Duration Offset Tbl vs Air Flow

;*

;*

;* Tbl = Sec's * 40

;* msec * 0.04

;* msec = N * 25

;*==================================================

LD3FD:

;----------------------------------

; 	msec 		; Air Flow gms/sec

;----------------------------------

FCB 10		;  0

FCB  6		; 16

FCB  4		; 32

FCB  2		; 48

FCB  1		; 64

;*==================================================

;* Proportional Gain Flow Factor vs Air Flow

;*

;* Used to modify prop gain count.

;*

;* Tbl = Mult * 256

;*==================================================

LD402:

;----------------------------------

; 	Mult 		; Air Flow

;----------------------------------

FCB  16		;   0

FCB  18		;  16

FCB  40		;  32

FCB  64		;  48

FCB  80		;  64

;*==================================================

;* Int Delay Mult vs Slow O2 Error

;*

;* Integrator Delay As Funct Of Slow O2

;*

;* Mult x Int Delay Mult (vs. Slo O2) * LIntDly

;*

;* Tbl = Factor * 256

;*==================================================

LD407:

;----------------------------------

; 	mult 		; O2 Error

;----------------------------------

FCB 255		;   0

FCB 255		;   8

FCB 255		;  16

FCB 224		;  24

FCB 192		;  32

FCB 160		;  40

FCB 128		;  48

FCB  96		;  56

FCB  80		;  64

FCB 128		;  72

FCB 160		;  80

FCB 192		;  88

FCB 192		;  96

;-----------------------------------------------

; ALDL list

;-----------------------------------------------

;----------------------------

LD414:	FDB $D002		; 1. eprom id lsb

LD416:	FDB $D003		; 2. eprom id msb

;----------------------------

LD418:	FDB $00FD		; 3. current IAC posit, (0-255)

; n = counts

;----------------------------

LD41A:	FDB $0023		; 4. coolant

; see table 1

;----------------------------

LD41C:	FDB $0030		; 5. mph,

; (mph/1)

;----------------------------

LD41E:	FDB $002A		; 6. map, (a/d, 0-255)

; volts = n * .0196

;----------------------------

LD420:	FDB $001B		; 7. rpm/25

; rpm = n * 25

;----------------------------

LD422:	FDB $0042		; 8. tps (a/d, 0-255)

; volts = n * .0196

;----------------------------

LD424:	FDB $00A2		; 9. integrator, (0-255)

;----------------------------

LD426:	FDB $0039		; 10. O2 (a/d), mvdc = 0.2304 * a/d val??

;	mv = N * (1000/226) = N * 4.425??

;----------------------------

LD428:	FDB $00E1		; 11. error flag 1

;

; b7 error code 12 no drp

; b6 error code 13 oxygen sensor

; b5 error code 14 coolant sensor hi

; b4 error code 15 coolant sensor low

;

; b3 error code 21 tps high

; b2 error code 22 tps low

; b1 error code 23 iat sensor lo

; b0 error code 24 vss

;----------------------------

LD42A:	FDB $00E2		; 12. error flag 2

;

; b7 error code 25 iat sensor hi

; b6 error code 31 governor fail

; b5 error code 32 not used (egr)

; b4 error code 33 map sensor high

;

; b3 error code 34 map sensor low

; b2 error code 35 not used

; b1 error code 41 not used

; b0 error code 42 est monitor error

;----------------------------

LD42C:	FDB $00E3		; 13. error flg 3

;

; b7 error code 43 knock spark control failure

; b6 error code 44 O2 lean

; b5 error code 45 O2 rich

; b4 error code 51 prom error

;

; b3 error code 55 adu error, netres/RTS ovrrun

; b2 error code 54 not used

; b1 error code 53 fuel pump relay failure

; b0 error code 52 cal pack missing

;--------------------------------------

LD42E:	FDB $000E		; 14. a/f mode word flag 1, (flag),

;

; b7 closed loop flag, 1 = closed loop

; b6 rich/lean flag, 1 = rich

; b5 closed loop flag, 1 = closed loop

; b4 async fuel flag

;

; b3 4-3 downshift for tcc unlock

; b2 low battery, 1 = low

; b1 blm enable flag, 1 = enable store

; b0 clear flood flag, 1 = cranked in c/flood

;--------------------------------------

LD430:	FDB $0040		; 15 battery volts, (a/d, 0-255) vbatt/10

;--------------------------------------

LD432:	FDB $0801		; 16. mcu2 i/o

;

; b7 air conditioner, 0 = a/c requested

; b6 p/s pressure switch

; b5 high gear, 0 = switch open

; b4 park neutral, 0 = drive

;

; b3 tcc/shift light

; b2 a/c

; b1 air divert

; b0 select

;--------------------------------------

LD434:	FDB $0069		; 17. PA3, counts of msb only

;--------------------------------------

LD436:	FDB $00A1		; 18. BLM, (blm/1)

;--------------------------------------

LD438:	FDB $0024		; 19. IAT, a/d

;----------------------------

;*==================================================

;*

;* Table 1 --- Coolant Temperature, 1k pu

;*

;*

;* 	 A/D 	degrees		A/D

;* 	counts     C		Inv

;* 	------- -------		---

;* 	255	    -40		  0

;* 	251	    -30		  4

;* 	250	    -25		  5

;* 	247	    -20		  8

;* 	245	    -15		 10

;* 	241	    -10		 14

;* 	237	    - 5		 18

;* 	231	      0		 24

;* 	225	      5		 30

;* 	218	     10		 37

;* 	209	     15		 46

;* 	199	     20		 56

;* 	189	     25		 66

;* 	177	     30		 78

;* 	165	     35		 90

;* 	152	     40		103

;* 	139	     45		116

;* 	126	     50		129

;* 	114	     55		141

;* 	102	     60		153

;* 	92	     65		163

;* 	81	     70		174

;* 	72	     75		183

;* 	64	     80		191

;* 	56	     85		199

;* 	50	     90		205

;* 	44	     95		211

;* 	39	    100		216

;* 	34	    105		221

;* 	30	    110		225

;* 	26	    115		229

;* 	23	    120		232

;* 	21	    125		234

;* 	18	    130		237

;* 	16	    135		239

;* 	14	    140		241

;* 	13	    145		242

;* 	12	    150		243

;* 	0	    200		255

;*

;*==================================================

;* Pressure in KPa to Inches Mercury Vacuum

;*==================================================

;*

;* 	KPa		 "Hg

;* 	  0		   0

;* 	 10		 2.9

;* 	 20		 5.9

;* 	 30		 8.9

;* 	 40		11.8

;* 	 50		14.8

;* 	 60		17.7

;* 	 70		20.7

;* 	 80		23.7

;* 	 90		26.6

;* 	100		29.6

;*

;*==================================================

;*==================================================

;* Diagnostic Parameter Mask's

;*

;* 0 = Disable error recognition

;* 1 = Enable error recognition

;*

;*==================================================

LD43A:	FCB	$FF	; 1111 1111, 255

; b7 code 12, No ref's, (eng not running)

; b6 code 13, O2 sensor

; b5 code 14, Coolant Sensor Hi

; b4 code 15, Coolant sensor Lo

;

; b3 code 21, TPS hi

; b2 code 22, TPS lo

; b1 code 23, IAT Sensor Lo

; b0 code 24, VSS

;

LD43B:	FCB	$B9	; 1011 1001, 185

; b7 code 25, IAT Sensor Hi

; b6 code 31, MAP LOW, (NOT USED)

; b5 code 32, EGR failure,

; b4 code 33, MAP Sensor hi

;

; b3 code 34, MAP Sensor low

; b2 code 35, IAC error, (NOT USED)

; b1 code 41, NO DRP , (NOT USED)

; b0 code 42, EST Mon error

LD43C:	FCB	$F3	; 1111 0011, 243

; b7 code 43, ESC failure

; b6 code 44, O2 Lean

; b5 code 45, O2 Rich

; b4 code 51, EPROM error

;

; b3 code 52, Cal-pack missing, (NOT USED)

; b2 code 53, VATS, (NOT USED)

; b1 code 54, Fuel pump relay malfunction

; b0 code 55, ADU Error, netres/RTS ovrrun

;----------------------------------------------

LD43D:	FCB	 50	; good startups to erase error codes

;----------------------------------------------

; Error Log Time Constants

;----------------------------------------------

LD43E:	FCB	 10	; error log time constant 1, 1 sec

LD43F:	FCB	 20	; error log time constant 2, 2 sec

LD440:	FCB	100	; error log time constant 3, 10 sec

LD441:	FCB	120	; error log time constant 4, 12 sec

;----------------------------------------------

; Error 13 param's

; O2 Sensor

;----------------------------------------------

LD442:	FCB	 60	; Time since run eable, (120 Sec)

LD443:	FCB	 81	; 352 mvdc, O2 Sensor Lo limit O2 Test window

LD444:	FCB	127	; 552 mvdc, O2 Sensor Hi limit

;

LD445:	FCB	 13	; TPS Min limit, (5%)

LD446:	FCB	 30	; Time limit, (60 Sec)

;----------------------------------------------

; Error 13/14 param's

; O2/map

;----------------------------------------------

LD447:	FCB	 81	; 65c, Coolant Min thresh, 1k PU

;----------------------------------------------

; Error 14 param's

; Hi Coolant

;----------------------------------------------

LD448:	FDB	  2	; eng run time val for coolant high diag test, 2 sec

LD44A:	FCB	 12	; high coolant error limit, inverse

;----------------------------------------------

; Error 15 param's

; Lo Coolant

;----------------------------------------------

LD44B:	FDB	 30	; eng run time val for coolant low diag test, 30 sec

LD44D:	FCB	252	; low coolant error limit, inverse

;----------------------------------------------

; Error 14/15 param

; Coolant default

;----------------------------------------------

LD44E:	FCB	100	; coolant value on diag error

;----------------------------------------------

; Error 21 param's

; Tps High

;----------------------------------------------

LD44F:	FCB	128	; 2.5vdc, TPS limit,

LD450:	FCB	 80	; 8 Sec, Time limtit,

LD451:	FCB	112	; 51.7 Kpa Baro limit,

;----------------------------------------------

; Error 22 param's

; Tps Low

;----------------------------------------------

LD452:	FCB	 10	; tps limit, 0.20 vdc

LD453:	FCB	 80	; default tps, 0.?? vdc (~??%)

;----------------------------------------------

; Error 23 param's

; Lo iat

;----------------------------------------------

LD454:	FCB	250	; low iat error limit, inverse

LD455:	FDB	301	; min engine run time for valid test

LD457:	FCB	 35	; mph diag threshold

LD458:	FCB	152	; default iat on diag error

;----------------------------------------------

; Error 24

; Vss

;----------------------------------------------

LD459:	FCB	  5	; mph threshold

;

; valid test rpm window

LD45A:	FCB	 64	; rpm threshold

LD45B:	FCB	240	; rpm threshold

LD45C:	FCB	 40	; map adc threshold

LD45D:	FCB	  8	; tps% threshold

LD45E:	FCB	  5	; 5 sec time limit

;----------------------------------------------

; Error 25 param's

; Hi iat

;----------------------------------------------

LD45F:	FCB	 16	; high iat error limit, inverse

;----------------------------------------------

; Error 32 param's

; EGR

;----------------------------------------------

LD460:	FCB	 10	;

LD461:	FCB	 80	;

LD462:	FCB	144	;

LD463:	FCB	 23	;

LD464:	FCB	 51	;

LD465:	FCB	 10	;

LD466:	FCB	  3	;

LD467:	FCB	  1	;

LD468:	FCB	 25	;

LD469:	FCB	  8	;

LD46A:	FCB	 50	;

LD46B:	FCB	  1	;

LD46C:	FCB	 12	; SA multiplier for L008C, 4.2deg

LD46D:	FCB	128	; filt coef for L008C

LD46E:	FCB	 32	; filt coef for L008C

LD46F:	FCB	 40	;

LD470:	FCB	 19	;

;----------------------------------------------

; Error 33 param's

; Map High

;----------------------------------------------

LD471:	FCB	 10	; ?% TPS limit

LD472:	FCB	156	; ??.0 Kpa, BARO max limit

LD473:	FCB	 50	; 5 Sec's Time limit

;----------------------------------------------

; Error 34 param's

; Map Low

;----------------------------------------------

LD474:	FCB	  5	; map limit, (11.9 kpa)

LD475:	FCB 	 64	; 1200 rpm, (64 * 12.5 + 400)

LD476:	FCB	  2	; 0.05 sec time limit (sec * 80)

LD477:	FCB	 54	; 21% tps

; ----------------------

LD478:	FCB	218	; 90.8 kpa default map/baro (adc cnts)

; max adc map on error

;----------------------------------------------

; Error 33/34 Param

; Map Default Coef

;----------------------------------------------

LD479:	FCB	146	; MAP default coef, COEF * 64, for tps value

;----------------------------------------------

; Map Bias Default vs Rpm

;----------------------------------------------

LD47A:

FCB 64		; upper tbl limit

;----------------------------------

; 	a/d val		; rpm

;----------------------------------

FCB 91		;  800

FCB 45		; 1600

FCB 10		; 2400

FCB  0		; 3200

;----------------------------------------------

LD47F:	FCB	  6	; IAC test val

LD480:	FCB	145	;

LD481:	FCB	  0	;

;----------------------------------------------

; Error 42 param's

; EST error

;----------------------------------------------

LD482:	FCB	  4	; scaled rpm val

LD483:	FCB	  0	; Num PA1 cnt's for error

LD484:	FCB	  4	; Num of EST faults for 42A

LD485:	FCB	  4	; Num of EST faults for 42B

;----------------------------------------------

; Error 43 param's

; ESC error

;----------------------------------------------

LD486:	FDB	240	; knock sec's thres from eng run

LD488:	FCB	235	; runaway knock cnts for ESC err 43

LD489:	FCB	  5	; timer duration

LD48A:	FCB	 53	; 87c, coolant thres low

LD48B:	FCB	 12	; 150c, coolant thres high

LD48C:	FCB	160	; 4000 rpm, max rpm thres

LD48D:	FCB	196	; 81.25 KPa, min map thres

LD48E:	FCB	 40	; added SA for knock test, 14 deg

LD48F:	FCB	 57	; added SA for knock test, 20 deg

LD490:	FCB	  2	; required knock cnts for diag

;----------------------------------------------

; Error 44

; O2 Lean

;----------------------------------------------

LD491:	FCB	 58	; 251mV, low limit

LD492:	FCB	 50	; diag timer val

;----------------------------------------------

; Error 45

; O2 Rich

;----------------------------------------------

LD493:	FCB	173	; 751 mvdc, O2 sensor hi limit

LD494:	FCB	 30	; diag timer, O2 rich

LD495:	FCB	 51	; 20%, tps hi limit

LD496:	FCB	  0	; 0%, tps low limit

;----------------------------------------------

LD497:	FCB	  0	; timer for error codes?

;----------------------------------------------

; Error 54

; Pump Relay Error

;----------------------------------------------

LD498:	FCB	 30	; fuel pump volts diag time value

;----------------------------------------------

;----------------------------------------------

LD499:	FCB	  2	; timer val

LD49A:	FCB	  1	; major loop O2 filter coef, for L003C

LD49B:	FCB	161	; O2 val, 699mV

LD49C:	FCB	 65	; O2 val, 282mV

LD49D:	FCB	 50	;

LD49E:	FCB	164	; scaled rpm val

LD49F:	FCB	 20	; counter/timer val, in L0077

LD4A0:	FCB	  8	; map val, 22.5 Kpa

LD4A1:	FCB	 80	; delta map val, 25 Kpa

LD4A2:	FCB	199	; coolant thres, 20c

LD4A3:	FCB	 50	; timer/counter for L0078

LD4A4:	FCB	  0	; timer/counter thres for L0079

LD4A5:	FCB	 96	; coolant thres, 67c

LD4A6:	FCB	255	; map thres

LD4A7:	FCB	255	; rpm / 25 val, 6375

LD4A8:	FCB	 38	; rpm / 25 val, 950 rpm

LD4A9:	FCB	  1	; timer/counter thres for L007F

; ----------------------

; hyst pairs

; ----------------------

LD4AA:	FCB	  2	; mph

LD4AB:	FCB	  1	; mph

LD4AC:	FCB	  8	; tps%

LD4AD:	FCB	  5	; tps%

; ----------------------

LD4AE:	FCB	205	;

;----------------------------------------------

;----------------------------------------------

LD4AF:	FCB	123	; int val, egr tst?

LD4B0:	FCB	116	; int val, egr tst?

LD4B1:	FCB	  3	; egr test val?

LD4B2:	FCB	 16	; egr test val?

LD4B3:	FCB	  5	; egr?

LD4B4:	FCB	 20	; mph thres

LD4B5:	FCB	112	; blm thres

;-----------------------------------------------

; TCC

;-----------------------------------------------

LD4B6:	FCB	102	; 60c, min for tcc lookup

; indexed table entries

LD4B7:	FCB	  3	;

LD4B8:	FCB	 40	; mph thres

LD4B9:	FCB	 10	; tps limit

LD4BA:	FCB	 21	; tps limit

LD4BB:	FCB	  4	; tps limti hyst

LD4BC:	FCB	  0	;

LD4BD:	FCB	128	; filter coef for MPH / 1, L0030

LD4BE:	FCB	  1	; lk/unlk? delay val

LD4BF:	FCB	  6	; re-lock delay val

LD4C0:	FCB	 80	; mph thres for mandatory lockup of tcc

;-----------------------------------------------

; TCC

;-----------------------------------------------

LD4C1:	FCB	38 255  ; lk > mph

LD4C3:	FCB	36 255	; unlk < mph

;-----------------------------------------------

; 2d lkup's w/SBA

;

; tps% req TCC relock/unlock vs. mph

;-----------------------------------------------

; mph:   20  24  28  32  36  40  44  48  52  56  60

LD4C5:	FCB	  2   2  19  58  68  72  78  82 104 120 120

LD4D0:	FCB	  1   1   8  10  27  37  45  52  64  90  96

;-----------------------------------------------

;-----------------------------------------------

LD4DB:	FCB	 32 255	; lk > mph

LD4DD:	FCB	 30 255	; unlk < mph

;-----------------------------------------------

; 2d lkup's w/SBA

;

; tps% req TCC relock/unlock vs. mph

;-----------------------------------------------

; mph:   20  24  28  32  36  40  44  48  52  56  60

LD4DF:	FCB	  2   2  10  56  74  88  97 112 120 120 120

LD4EA:	FCB	  1   1   8  22  41  51  69  84  90  90  96

;*==================================================

;* Tps Mult vs. Baro

;*

;* tbl = factor * 255

;*==================================================

; 	MULT 		; Kpa BARO

;----------------------------------

LD4F5:

FCB 255		;  75

FCB 255		;  85

FCB 255		;  95

FCB 255		; 105

;*==================================================

;*

;* Shift Light Parameters

;*

;* Indexed tbl + 19

;*==================================================

LD4F9:	FCB	160	; 1st gr n/v

FCB	 99	; 2nd gr n/v

FCB	 65	; 3rd gr n/v

FCB	 49	; 4th gr n/v**

FCB	100	;

FCB	 66	;

FCB	 50	;

LD500:

FCB	 35	;

FCB	 38	;

FCB	 39	;

FCB	 42	;

FCB	 51	;

FCB	128	;

FCB	128	;

FCB	128	;

FCB	128	;

FCB	209	;

FCB	 13	;

FCB	 16	;

FCB	176	;

; ----------------------

LD50D:	FCB	 10	;

LD50E:	FCB	 15	;

;*==================================================

;*

;* IAC Parameters

;*

;*==================================================

LD50F:	FCB	 40	;

LD510:	FCB	 18	;

LD511:	FCB	160	; IAC park position

; ----------------------

; Stall Saver

; ----------------------

LD512:	FCB	 38	; 475 rpm, rpm / 12.5, go into stall saver

LD513:	FCB	 52	; 650 rpm, rpm / 12.5, exit stall saver

; ----------------------

LD514:	FCB	  5	; 2%, tps% threshold, max for idle adjust allowed

;---------------------------------------------

; Indexed Tbl

;

; Timer delay before changing IAC for idle

;---------------------------------------------

LD515:	FCB	  3	; small error retract, idle too lo

LD516:	FCB	  1	; large error retract, idle too lo

LD517:	FCB	  2	; small err extend, idle too hi

LD518:	FCB	  0	; large err extend, idle too hi

;---------------------------------------------

; These two are rpm breakpoints for desired idle rpm IAC changes

LD519:	FCB	  3	; neg err bk pt for large gain wd (rpm / 12.5)

LD51A:	FCB	  4	; pos err bk pt for large gain wd (rpm / 12.5)

; ----------------------

; ----------------------

LD51B:	FCB	  2	; 25 rpm, stall saver hyst?

LD51C:	FCB	  3	;

LD51D:	FCB	200	; retract pulse steps

LD51E:	FCB	 22	; non-vol ram fail init for L00E4

LD51F:	FCB	 87	; 8.7 vdc, stepper motor protect

LD520:	FCB	 16	;

LD521:	FCB	  9	;

LD522:	FCB	  1	; idle control delay for in gear, rpm > 900 rpm

LD523:	FCB	 60	;

; ----------------------

LD524:	FCB	 36	; 102c, coolant threshold for added A/C IAC steps

LD525:	FCB	  5	; added A/C steps

LD526:	FCB	  3	; added steps for ??

LD527:	FCB	 65	;

LD528:	FCB	  2	;

LD529:	FCB	  0	;

LD52A:	FCB	 45	; 562.5 rpm, added to idle rpm for low BatV?

LD52B:	FCB	 80	; 1000 idle rpm for aldl mode

LD52C:	FCB	 44	; 550 rpm, desired idle speed

LD52D:	FCB	  1	; added to IAC w/low BatV

LD52E:	FCB	 12	; max added to IAC w/low BatV

LD52F:	FCB	250	; mph threshold

LD530:	FCB	126	; BatV thres, 12.6 volts

LD531:	FCB	 16	; delay val for low BatV changes

LD532:	FCB	191	; Max IAC steps

LD533:	FCB	 90	;

LD534:	FCB	 56	;

LD535:	FCB	112	; rpm / 12.5, 1400 rpm

LD536:	FCB	 72	; rpm / 12.5, 900 rpm

LD537:	FCB	  2	;

LD538:	FCB	 96	; max TF steps

LD539:	FCB	 32	; TF decay filt coef, < low mph

LD53A:	FCB	 16	; TF decay filt coef, > low mph & < high mph

LD53B:	FCB	 25	; TF decay filt coef, > high mph

LD53C:	FCB	 64	; filt coef

LD53D:	FCB	 15	; filt coef

LD53E:	FCB	  5	; TF decay low mph thres, between filt coef @ D539 & D53A

LD53F:	FCB	 20	; TF decay high mph thres, between filt coef @ D53A & D53B

LD540:	FCB	 10	; min TF steps if mph > 5

LD541:	FCB	  6	; 2.3%, TF tps% bias

LD542:	FCB	 32	; TF factor in prk/neut

LD543:	FCB	 64	; TF factor not in prk/neut

LD544:	FCB	  4	;

LD545:	FCB	156	;

LD546:	FCB	 25	; mph threshold, IAC bias

;*==================================================

;* IAC Motor Position vs Coolant

;*

;* Tbl = Steps

;*==================================================

LD547:

;---------------------------------

; 	steps  		; Deg c Deg f

;---------------------------------

FCB 200		; COLD 	COLD

FCB 190		; -8.5 	17

FCB 154		;  6.0 	43

FCB 140		; 15.5 	60

FCB 131		; 23.5 	74

FCB 125		; 30.5 	87

FCB 115		; 37.0 	99

FCB 100		; 43.3 	110

FCB  85		; 49.3 	121

FCB  75		; 56.0 	133

FCB  68		; 63.0 	145

FCB  66		; 71.0 	160

FCB  66		; 80.0 	176

FCB  64		; 91.0 	196

FCB  65		; 107.	225

FCB  65		; 136.	277

FCB  65		; HOT 	HOT

;*==================================================

;* Position After Motor Reset vs Baro

;*

;* Tbl = Steps

;*==================================================

LD558:

;---------------------------------

; 	STEPS 		; Kpa BARO

;---------------------------------

FCB 20		; 75

FCB 18		; 85

FCB 15		; 95

FCB 10		; 105

;*==================================================

;* A/C requested additional steps

;*

;* Tbl = Steps

;*==================================================

LD55C:

;---------------------------------

; 	STEPS 		; Kpa BARO

;---------------------------------

FCB  4		; 75

FCB  2		; 85

FCB  0		; 95

FCB  0		; 105

;*==================================================

;* Additional IAC Motor Position vs Coolant

;*

;* Tbl = Steps

;*==================================================

LD560:

;---------------------------------

; 	steps  		; Deg c Deg f

;---------------------------------

FCB   1		; COLD 	COLD

FCB   1		;  6.0 	43

FCB   1		; 23.5 	74

FCB   1		; 37.0 	99

FCB   1		; 49.3 	121

FCB   1		; 63.0 	145

FCB   1		; 80.0 	176

FCB   1		; 107.	225

FCB   3		; HOT 	HOT

;-----------------------------------------------

;

; Start of Code

;